NAVAIS GNSS RECEIVER

NAVAIS GNSS receiver are designed to deliver high-precision positioning, powered by Septentrio’s technology which is Based on multi-frequency GNSS tracking and advanced Real-Time Kinematic (RTK) positioning processing.

By tracking all major satellite constellations (GPS, Galileo, GLONASS, BeiDou, QZSS, and NavIC) and leveraging advanced Real-Time Kinematic positioning algorithms, our GNSS receiver maximise signal availability, positioning accuracy, and reference network compatibility in challenging environments.

- Complete multi-frequency, multi-constellation GNSS receiver: GPS, GLONASS, Galileo, BeiDou, QZSS

- Precise and reliable cm-level RTK positioning with signal health and integrity indicators

- Best-in-class anti-interference, anti-jamming, anti-spoofing technology: Advanced Interference Monitoring and Mitigation (AIM+)

- Easy-to-integrate with survey, marine and construction applications and hardware

- Comes with intuitive web UI for easy operation & monitoring.

- Integrated with computer system and 4G industrial router.

- Multiple outputs RS232, RS485, TCP, UDP, NETWORK, NMEA-0183, RTCM CMR etc.

- IONO+: the most advanced protection against ionospheric disturbances

- Advanced Receiver Autonomous Integrity Monitoring (RAIM+)

| Technical Specifications GNSS Performance - GNSS Engine: Septentrio™ - Channels: 448 hardware channels Signals Tracked Simultaneously: - GPS: L1C/A, L1PY, L2C, L2P, L5 - GLONASS: L1CA, L2CA, L2P, L3 CDMA - Galileo: E1, E5a, E5b, E5 AltBOC, E6 - BeiDou: B1I, B1C, B2I, B2a, B2b, B3I Update Rate: Up to 100 Hz Time to First Fix (TTFF): - Cold Start: < 45 seconds - Warm Start: < 20 seconds - Signal Re-acquisition: < 1 second | Positioning Accuracy - RTK (Horizontal): 0.6 cm + 0.5 ppm - RTK (Vertical): 1 cm + 1 ppm - SBAS (Horizontal): 0.6 m - DGPS (Horizontal): 0.4 m - Velocity Accuracy: 0.05 m/s Communications & Protocols - Supported Data Formats: - RTCM v2.x, RTCM v3.x (including MSM), CMR, CMR+, NTRIP - Output Formats: - NMEA-0183 (v2.3, v3.01, v4.1), Septentrio Binary Format (SBF) - Hardware Interfaces: - USB 2.0 (Device & Host mode) - RS232 Out - RS485 Out - PPS out - Event in - Ethernet (LAN and WAN) |
| --- | --- |
| Integrated Solution - 4G Network Industrial router - Compact PC and monitor Environmental Attributes - Operating Temperature: -40°C to +85°C - Storage Temperature: -55°C to +85°C - Humidity: 95% non-condensing | Physical & Electrical - Dimensions: 200 mm × 200 mm × 130 mm - Weight: < 500 grams - Input Voltage: 12V DC 5A - Antenna Connector: TNC - PPS connector: BNC - 4G and Wifi connector: SMA |